[’26 FE] Design of Secure Remote Diag on Container Separation Architecture

Table of Contents

1. About This Document	3

2. Introduction	4

2.1. Purpose	4

2.2. Audience	4

2.3. Abbreviations / Terms	4

2.4. Related Documents	5

3. Project context	6

3.1. Software context	6

3.2. Software Architectural Driver	6

3.3. Background	7

4. 3	Problem Defined	7

4.1. Functional Requirements	7

4.2. Problem Identification	8

4.2.1. Problem Description	8

4.2.2. Problem Analysis	8

4.3. Quality Attributes	9

5. 4	Architecture Alternatives	9

5.1. Alt-A: Split RemoteDiag	9

5.2. Alt-B: Move RemoteDiag	11

5.3. Alt-C: Move RemoteDiag + Proxy	12

5.4. Comparison and decision	15

6. 5	Implementation and Verification	16

6.1. Implementation	16

6.1.1. Forward API call from proxy to remotediag	16

6.1.2. Filter scenario	17

6.2. Verification	17

6.2.1. SidFilter blocks reprogramming diagnostic requests	17

6.2.2. RemoteDiagProxy and RemoteDiag communication	17

6.3. Quality Attributes result	18

7. 6	Conclusion	18

8. Appendix	19

8.1. Diagnostic filtering requirements	19

List of figures

Figure 1 - Current architecture	6

Figure 2 - New architecture (containerized)	6

Figure 3 - RemoteDiag communication paths	7

Figure 4: Alt-A — Split RemoteDiag	9

# About This Document

Document Information

| Document Title | SW Design |
| --- | --- |
| Project | 24DCM LM |
| Issuing Authority | Toyota |
| Author | Vu Hoang Phi (phi.vu@lge.com) |
| Status of Document | Completed |

Table 1- Document information

Revision History

| Version | Date | Content of Change | Author | Reviewer |
| --- | --- | --- | --- | --- |
| 01 | 2026.09.28 | Initial Release | phi.vu | by.kim |

Table 2- Revision history

# Introduction

## Purpose

This document presents the software design for Secure Remote Diag on Container Separation Architecture in the Toyota 24DCM LM telematics project. Due to security requirements, the OEM requires the DCM to be separated into two partitions using LXC containers. The document identifies the security risks of the current RemoteDiag communication paths under the new containerized architecture, analyzes the required quality attributes, evaluates multiple architecture alternatives, and provides a comprehensive design solution that blocks unauthorized ECU reprogramming commands and keeps MM ECU diagnostic traffic entirely within the internal container.

## Audience

The readers of this article are as follows:

- Software Architect

- Developer

- Project Leader

- Project Manager

- Test Engineer

## Abbreviations / Terms

| Abbreviation | Description |
| --- | --- |
| DCM | Data Communication Module |
| OEM | Original Equipment Manufacturer |
| LXC | Linux Containers |
| EP | Entry Point (container): contains externally exposed network components |
| IP | Internal Partition (container): contains components with access to the CAN bus |
| OTA | Over-The-Air (update) |
| UDS | Unified Diagnostic Services / Unix Domain Socket (per context) |
| SID | Service Identifier (UDS diagnostic service ID) |
| RDP | RemoteDiagProxy |
| OBC | OnBoardClient |
| MM | MM ECU (OTA Master) |
| CAN | Controller Area Network |
| IPC | Inter-Process Communication |
| ECU | Electronic Control Unit |

Table 3- Abbreviations

## Related Documents

| ID | Title | Version |
| --- | --- | --- |
| 19PFv3KAI-DCM_Estimation_Cybersecurity_Specification_20251203 | The base Cybersecurity Requirement Specifications and Additional requirements for 19PFv3KAI | -- |
| SEC-ePF-MFG-REQ-SPEC | In-Vehicle Network Requirements Specification of Message Filtering | a03-00-a |

Table 4- Related documents

# Context

## Software context

Toyota DCM is a telematics platform providing connected vehicle services, including remote diagnostics from the OEM server. It supports diagnostic communication and OTA updates through the Internal CAN bus.

The current architecture follows a monolithic deployment model, with all components running in a single partition.

Figure 1 - Current architecture

## New Architecture change

Due to security, the OEM wants to separate DCM into 2 partitions using LXC containers:

- EP container (Entry Point): externally exposed network components.

- Internal container (IP): components which have access to the CAN bus.

Figure 2 - New architecture (containerized)

# 3	Problem

## Functional requirements

| # | Description |
| --- | --- |
| FR-01 | The system shall block unauthorized diagnostic commands related to ECU reprogramming. |
| FR-02 | The system shall route UDS diagnostic messages exchanged with the MM ECU entirely within the IP. |

## Problem identification

### Problem description

RemoteDiag is a module that processes diagnostics for each ECU and uploads diagnostic data to the OEM Server. It has 2 communication paths:

- with OEM Server: performs direct diagnostic commands.

- with MM ECU: OTA diagnostic update.

- Risk: the RemoteDiag ↔ MM path routes through the EP, which is insecure.

The OEM requests that unauthorized reprogramming requests should be filtered.

RemoteDiag’s internal architecture and communication should be redesigned to align with the new architecture.

Figure 3 - RemoteDiag communication paths

### Problem Analysis

The EP container hosts externally exposed network components (MqttMgr, HttpMgr and OEM applications). If the EP container is compromised, an attacker can inject reprogramming diagnostic requests toward the internal CAN bus through RemoteDiag’s existing communication paths.

The MM ECU OTA diagnostic flow (TCP) currently terminates in the EP container, violating the requirement that diagnostic traffic with the MM ECU must be handled entirely within the IP container.

Therefore the design must (1) filter reprogramming-related SIDs before they reach the internal CAN bus, and (2) relocate the MM ECU diagnostic endpoint into the IP container, while minimizing the impact on the 10+ existing EP-side manager modules that communicate with RemoteDiag over binder IPC.

## Quality Attributes

The following quality attributes were carefully considered and prioritized while redesigning RemoteDiag. These attributes guided the architectural decisions and solution selection:

Table 5: Quality Attributes

| ID | QA Scenario | Measure | Type | Priority |
| --- | --- | --- | --- | --- |
| QA-01 | A compromised app/service sends reprogramming diagnostic to CAN | Zero Reprogramming SIDs reaches the internal CAN. | Security | High |
| QA-02 | MM ECU sends a diagnostic request via TCP to RemoteDiag | Zero diagnostic communication is found in EP. 100% of requests are handled within IP. | Security | High |
| QA-03 | The new design preserves the original core responsibility of each component. | No component is required to take responsibilities outside its originally defined role. No additional specification updates caused by responsibility expansion. | Modifiability | High |
| QA-04 | Existing software components (other than RemoteDiag) require minimal changes. | Number of components which need to be modified to align with the new design. | Maintainability | Medium |

# Architecture Alternatives

## Alt-A: Split RemoteDiag

Split RemoteDiag responsibilities into 2 apps:

- RemoteDiag (EP): handles diag messages from OEM server.

- RemoteOTA (IP): handles OTA diag message flows from MM ECU.

- OnBoardClient is responsible for reprogramming message filtering and diagnostic priority control.

Figure 4: Alt-A — Split RemoteDiag

## Alt-B: Move RemoteDiag

- Move RemoteDiag into the IP container.

- To maintain communication with the OEM server and other EP-side components, establish UDS socket connections to each module.

Trade-off — Maintainability: All EP modules require cross-container IPC support, increasing implementation effort and code-change scope.

Figure 5: Alt-B — Move RemoteDiag

## Alt-C: Move RemoteDiag + Proxy

- Move RemoteDiag into the IP container.

- Add a new RemoteDiagProxy (RDP) component in the EP that acts as a transparent proxy:

  - EP modules continue to interact with RDP using their existing interface.

  - RDP forwards requests to RemoteDiag via a UDS socket channel.

Figure 6: Alt-C — Move RemoteDiag + Proxy

## Comparison and decision

Consider the table to see the comparison of the designs:

Table 2: Comparison of architecture alternatives

|  | Alt-A – Split RemoteDiag | Alt-B – Move RemoteDiag | Alt-C – Move RemoteDiag + Proxy |
| --- | --- | --- | --- |
| QA-01 Security | High — Unauthorized diagnostic commands are filtered. | High — Unauthorized diagnostic commands are filtered. | High — Unauthorized diagnostic commands are filtered. |
| QA-02 Security | High — MM ECU diagnostic traffic remains within the EP container. (CyberSecurity team confirmed) | High — MM ECU diagnostic traffic remains within the EP container. (CyberSecurity team confirmed) | High — MM ECU diagnostic traffic remains within the EP container. (CyberSecurity team confirmed) |
| QA-03 Modifiability | Low — RemoteDiag and OnBoardClient’s responsibility is changed (diagnostic priority control is moved to OnBoardClient), requiring requirement updates. | High — Core responsibilities of existing components remain unchanged after migration. | High — Core responsibilities of existing components remain unchanged after migration. |
| QA-04 Maintainability | Medium — 01 additional component is modified (OnBoardClient — implements UDS socket server and priority control). | Low — 10 EP modules (PowerMgr, HttpMgr, …) have to implement cross-container UDS socket communication and fail-safe handling. | High — No changes required in existing modules. RemoteDiagProxy preserves the current binder interface. |
| Decision | Rejected | Rejected | Selected |

Rating: High = Fully satisfy · Medium = Partially satisfy · Low = Not satisfy

Key Decision Factors:

- Security: All three alternatives satisfy QA-01 and QA-02.

- Modifiability: Alt-B and Alt-C preserve the core responsibilities of the existing components; Alt-A changes the responsibilities of RemoteDiag and OnBoardClient.

- Maintainability: Alt-C requires no changes in existing modules — RemoteDiagProxy preserves the current binder interface, whereas Alt-B forces 10 EP modules to implement cross-container UDS socket communication.

Selected design: Alt-C — Move RemoteDiag + Proxy

# 5	Implementation and Verification

## Implementation

### Forward API call from proxy to remotediag

Callback transmission from the EP-side proxy to RemoteDiag in the IP container:

Figure 7: IPC callback transmission (proxy to remotediag)

API request-response from RemoteDiag back to the proxy:

Figure 8: IPC API request-response (remotediag to proxy)

The response payload format is [status: 1 byte][payload: N bytes], where status = 0x01 (OK) and status = 0x00 (ERROR). Callbacks are one-way (no response frame required).

### Filter scenario

The diagnostic filter scenario inside the IP container: RemoteDirectCommand validates each direct command and consults SidFilter before pushing the UDS request toward the internal CAN bus.

Figure 9: Diagnostic filter scenario

## Verification

### SidFilter blocks reprogramming diagnostic requests

Runtime log evidence — SidFilter rejects reprogramming-related SIDs before they reach the internal CAN bus:

3555985  66.2025  24LM  RDG  info  [ SidFilter.cpp : isAllowed : 45 ] SidFilter: blocked reprogramming-related request SID 11
3555986  66.2026  24LM  RDG  info  [ RemoteDirectCommand.cpp : validateDirectCommand : 344 ] Ecu list size = 1
3555987  66.2026  24LM  RDG  info  [ SidFilter.cpp : isAllowed : 45 ] SidFilter: blocked reprogramming-related request SID 28
3555989  66.2026  24LM  RDG  info  [ SidFilter.cpp : isAllowed : 45 ] SidFilter: blocked reprogramming-related request SID 85
3555991  66.2027  24LM  RDG  info  [ SidFilter.cpp : isAllowed : 45 ] SidFilter: blocked reprogramming-related request SID 10
3555993  66.2027  24LM  RDG  info  [ SidFilter.cpp : isAllowed : 45 ] SidFilter: blocked reprogramming-related request SID 34

### RemoteDiagProxy and RemoteDiag communication

Runtime log evidence — proxy IPC request/response across the UDS socket:

[ VehicleManagerAdapter.cpp : registerServiceLocked : 99 ] Registed Vehicle Manager Service with PID: 1333
[ VehicleManagerAdapter.cpp : registerServiceLocked : 101 ] Registed MsgInd_CanRx_MET1S02
...
[ ProxyIpcServer.cpp : requestAPICall : 219 ] ProxyIpcServer: request start request=VehicleGetTripCounter path=/dev/socket/remotediag/remotediag_proxy.sock timeoutMs=2000 maxRetries=10
[ ProxyIpcServer.cpp : requestAPICall : 247 ] ProxyIpcServer: sending request=VehicleGetTripCounter payloadLen=0 frameSize=8
[ ProxyIpcClient.cpp : dispatchRequest : 381 ] ProxyIpcClient dispatchRequest: command=VehicleGetTripCounter(9002) payloadLen=0
[ ProxyIpcServer.cpp : requestAPICall : 300 ] ProxyIpcServer: request complete request=VehicleGetTripCounter status=OK payloadLen=5

## Quality Attributes result

Table 3: Quality Attributes result

| Quality Attribute | Priority | Description | Result |
| --- | --- | --- | --- |
| QA-01 Security | High | Zero reprogramming SIDs reach the internal CAN — SidFilter blocks SID 0x10-02, 0x11, 0x28, 0x34, 0x85. | ✅ |
| QA-02 Security | High | MM ECU diagnostic requests are handled entirely within the IP container. | ✅ |
| QA-03 Modifiability | High | Core responsibilities of existing components remain unchanged after migration. | ✅ |
| QA-04 Maintainability | Medium | No changes required in existing modules; RemoteDiagProxy preserves the current binder interface. | ✅ |

# 6	Conclusion

The evaluation of three architecture alternatives (Split RemoteDiag, Move RemoteDiag, and Move RemoteDiag + Proxy) determined that Alt-C — moving RemoteDiag into the IP container together with a transparent RemoteDiagProxy in the EP container — provides the best balance of security, modifiability, and maintainability. Verification results confirm that both functional requirements are satisfied: SidFilter blocks all reprogramming-related SIDs (FR-01), and MM ECU diagnostic communication is handled entirely within the IP container (FR-02), while existing EP modules continue to use their unchanged binder interfaces through the proxy.

# Appendix

## Diagnostic filtering requirements

5.1. Diagnostic Filtering Targets —【MFGREQ_00005】

The ECU shall target the diagnostic request message excluding the legally applicable diagnostic communication addresses for diagnostic filtering. (For diagnostic communication addresses, refer to “Related Documents [2][10]”.)

(Supplement) “Reserve” in service tools and remote diagnostic response messages may be set in diagnostic request messages. Therefore, refer to the latest diagnostic address, and be careful not to exclude the diagnostic request message from the filtering target.

5.2. Diagnostic Filtering Implementation Details —【MFGREQ_00006】

The ECU shall discard diagnostic request messages in Table 5-1 by diagnostic filtering.

Table 5-1: Diagnostic requests to be filtered (Phase 5, 6)

| SID | Description |
| --- | --- |
| 0x10 | DiagnosticSessionControl — Programming session (0x02) |
| 0x11 | ECUReset |
| 0x28 | CommunicationControl |
| 0x34 | RequestDownload |
| 0x85 | ControlDTCSetting |

5.3. Diagnostic Filtering Deactivate Conditions —【MFGREQ_00007】

The ECU shall deactivate the diagnostic filtering of MFGREQ_00006 if the center connection device authentication completed successfully. For center connection device authentication, refer to “Related Documents [3]”.

5.4. Reactivating after Diagnostic Filtering deactivation —【MFGREQ_00016】

After the deactivation of the diagnostic filtering in【MFGREQ_00007】, the ECU shall reactivate the diagnostic filtering before the authentication state of the center connection device authentication transitions to “Unauthenticated” state. For the authentication state of the center connection device authentication, refer to “Related Documents [3]”.

TOYOTA MOTOR CORPORATION